- Please format the memory card firstly,and then unzip the file,put the 2 files (VT-RC02.bin and tx_update.txt) into the formatted memory card root directory.

- Place the memory card in the N4 and connect the rear camera to the N4 to connect the power supply.After power on,the machine will have a minute or so to identify the updated software (the screen may stay on a screen at this point),please wait patiently.

- After the software is detected,it will be updated and will take about 2-3 minutes,please wait patiently.

- After the software is updated successfully, it will return to the recording interface and start recording automatically. You can pause the video and go back to “System Setup”> “System Info” to view the dashcam version, as shown in the figure below.

- NOTE:

- Please update the V15 version firmware of the host camera first,and then update the rear lens firmware.

- After the update is completed, please remove the memory card,and put it in the computer to delete the update software, and then reboot the da shcam.

- If you don' t delete its' firmware after updating the rear lens firmware, the dashcam may be repeatedly restarted and the key may fail.